RISC-V Processor with Pipelining and Hazard Detection

Abstract

This project implements a five-stage pipelined RISC-V processor in Verilog, complete with hazard detection and forwarding logic. The design supports arithmetic, logical, load, store, and branch instructions. Pipeline registers are used between stages to enable instruction overlap, while hazard detection and forwarding units ensure correct execution in the presence of data dependencies. Simulation waveforms confirm correct instruction flow, register updates, and hazard resolution.

Introduction

Modern processors use pipelining to improve instruction throughput. However, pipelining introduces hazards that must be resolved to maintain correctness. This project demonstrates a RISC-V processor with:

- Five pipeline stages (IF, ID, EX, MEM, WB).

- Hazard detection for load-use hazards.

- Forwarding logic to minimize stalls.

- Instruction and data memory initialization for clean simulation.

Pipeline Stages

Stage 1: Instruction Fetch (IF)

- Program Counter (PC): Holds current instruction address.

- PC + 4 Adder: Computes sequential address.

- Instruction Memory: Provides instruction word.

- PC Selection MUX: Chooses between sequential PC and branch target.

- IF/ID Register: Stores instruction and PC+4 for next stage.

Stage 2: Instruction Decode (ID)

- Instruction Decoder: Extracts opcode, funct3, funct7, rs1, rs2, rd.

- Register File: 32 registers, read asynchronously, written in WB stage.

- Immediate Generator: Produces sign-extended immediate.

- Control Unit: Generates signals (RegWrite, MemRead, MemWrite, ALUSrc, MemToReg, Branch, ALUOp).

- ID/EX Register: Stores operands, immediate, and control signals.

Stage 3: Execute (EX)

- Operand Selection: ALUSrc decides between rs2_data and immediate.

- ALU Control: Uses ALUOp, funct3, funct7 to select operation.

- ALU: Performs arithmetic/logical operations.

- Branch Logic: Compares operands, calculates branch target, asserts branch_taken.

- EX/MEM Register: Stores ALU result, branch info, and control signals.

Stage 4: Memory Access (MEM)

- Data Memory: Accessed using ALU result as address.

  - Load: MemRead=1 → data forwarded.

  - Store: MemWrite=1 → data written.

- MEM/WB Register: Stores memory data, ALU result, and control signals.

Stage 5: Write Back (WB)

- WB MUX: Selects between ALU result and memory data (MemToReg).

- Register File Write: Updates destination register if RegWrite=1.

Hazard Detection and Forwarding

- Hazard Unit: Detects load-use hazards. If EX stage has a load and ID stage needs its result, PC and IF/ID are stalled, and ID/EX is flushed (bubble inserted).

- Forwarding Unit: Resolves data hazards by forwarding results from MEM/WB or EX/MEM back to EX stage inputs, reducing stalls.

Simulation Results

- Waveforms: Show PC incrementing, instructions fetched, ALU results computed, and register file updated.

- Hazard Handling: id_ex_flush asserts during load-use hazard, stalling pipeline for one cycle.

- Forwarding: fwdA/fwdB signals toggle when dependencies exist, ensuring correct operand values.

- Final Register Values: x1=5, x2=10, x3=15, x4=5, x5=0, x6=15, x7=25 — matching expected program behavior.

Conclusion

This project successfully demonstrates a pipelined RISC-V processor with hazard detection and forwarding. The design highlights key concepts in modern processor architecture: instruction overlap, hazard resolution, and control signal propagation. Simulation confirms correctness and provides insight into pipeline behavior.

Future Work

- Extend instruction set (e.g., shift, multiply).

- Add branch prediction to reduce stalls.

- Implement caches for realistic memory hierarchy.